TerrainFlow

QGIS Plugin — Current State Document

What is built, how it works, and what each part does

March 2026  |  For use alongside Claude Code sessions

# 1.  Purpose of This Document

This document captures the current built state of the TerrainFlow QGIS plugin. It is intended as a reference to return to when starting a new Claude Code session, picking up after a break, or troubleshooting a specific part of the plugin. It describes what exists, how it is structured, and what each component does — not what is planned for the future.

Use this document at the start of a Claude Code session by sharing it as context. This allows Claude Code to understand the existing codebase without needing to re-read every file.

# 2.  Plugin Overview

TerrainFlow is a QGIS docked panel plugin for land managers and permaculture designers. It enables terrain drainage analysis, rainfall runoff modelling, and interactive planning of water-retaining earthworks including swales, berms, detention basins, and dams.

The plugin runs inside QGIS 3.40 LTR as a docked side panel. All analysis is triggered from the panel and results are displayed as styled layers in the QGIS map canvas.

# 3.  File Structure

All plugin files live inside the plugin/ folder. The structure is:

plugin/

├── __init__.py

├── terrain_flow.py                # Main controller — all logic, signals, layer management

├── terrain_flow_dialog.py          # QDockWidget panel — all UI widgets

├── earthwork_properties_dialog.py  # Popup shown after drawing an earthwork

├── processing/

│   ├── earthwork.py               # Earthwork + EarthworkManager data classes, capacity formulas

│   ├── dem_burner.py              # Burns earthworks into DEM arrays; calculates ponding

│   ├── flow_analysis.py           # pysheds wrapper — flow direction, accumulation, catchments

│   └── scs_runoff.py              # SCS curve number runoff model, storm presets

└── map_tools/

├── draw_line_tool.py          # Left-click vertices, right-click/double-click to finish

├── draw_polygon_tool.py       # Same pattern for polygons

├── place_point_tool.py        # Single-click point placement (spillways)

├── ponding_query_tool.py      # Click a ponding zone to measure volume

└── select_contour_tool.py     # Click a contour line; reads elevation field

# 4.  Panel Sections & Widgets

The plugin panel is a QDockWidget divided into five sections. Each section contains specific widgets listed below with their widget names as used in the code.

## 4.1  Data

- combo_dem_layer — raster layer picker; auto-loads DEM on selection

- btn_load_dem — browse for DEM file from disk

- lbl_dem_status — displays loaded filename

- combo_boundary_layer — polygon layer for site boundary and exit point detection

## 4.2  Rainfall

- combo_preset — storm preset dropdown (from SCSRunoff.STORM_PRESETS); locks intensity and duration when selected

- spin_intensity — rainfall intensity in mm/hr

- spin_duration — storm duration in hours

## 4.3  Baseline Analysis

- spin_threshold_ha — flow threshold in hectares (0.1–500); lbl_channel_type updates live with channel description

- _btn_advanced / _widget_advanced — collapsible advanced section containing:

- spin_cn — SCS Curve Number (1–100, default 70)

- combo_moisture — Dry / Normal / Wet & Saturated

- combo_threshold_mode — Contributing area (ha) or Runoff volume (m³)

- spin_volume_threshold — m³ threshold, shown only when volume mode is selected

- chk_catchments — delineate catchment polygons (computationally slow, optional)

- btn_run — Run Baseline Analysis (disabled until DEM is loaded)

- progress_bar, lbl_status — progress and status feedback

## 4.4  Earthworks

- btn_draw_swale, btn_draw_berm, btn_draw_basin — drawing tools row 1

- btn_draw_dam — Draw Dam / Valley Wall tool

- combo_contour_layer — line layer picker; auto-set after contour generation

- btn_contour_swale — click a contour line to place a swale along it

- btn_draw_exclusion — activate exclusion zone polygon drawing

- btn_clear_exclusion_zones — remove all drawn exclusion polygons

- btn_toggle_exclusion_zones — show/hide exclusion zone layer

- btn_place_spillway — place a spillway point on an earthwork

- list_earthworks — checkable list of drawn earthworks; check = enabled; double-click to edit

- btn_edit_earthwork, btn_delete_earthwork — edit or remove selected earthwork

- btn_reanalyse — Re-analyse with Earthworks (re-burns DEM, re-runs flow analysis)

- btn_contours — Generate Contours using GDAL at user-specified interval

- btn_optimal_swale_contours — find best N contour elevations for swale placement

- btn_analyse_contour_flow — sample flow accumulation along contours; mark top 5

- spin_optimal_contour_count — number of optimal contours to find (1–10)

- lbl_contour_flow_result — displays contour flow analysis summary

- btn_toggle_before_after — toggle map between pre- and post-earthwork layers

## 4.5  Results

- btn_toggle_slope_class — show/hide semi-transparent slope classification overlay

- Inline slope legend swatches showing four zones (0–3°, 3–8°, 8–15°, >15°)

- btn_toggle_streams — show/hide stream network layer

- btn_toggle_accumulation — show/hide flow accumulation raster

- btn_toggle_slope_arrows — show/hide slope direction arrows

- btn_query_ponding — activate ponding query tool (click a ponding zone to measure volume)

- lbl_ponding_result — displays ponding volume result

- txt_results — runoff summary text output

# 5.  Earthwork Types

Four earthwork types are supported. Each is drawn differently, modifies the DEM differently, and has a different capacity calculation.

| Type | Drawn As | DEM Burn Method | Capacity Formula |
| --- | --- | --- | --- |
| Swale | Polyline | Lower cells by depth | Trapezoidal cross-section × length × 0.8 |
| Berm | Polyline | Raise cells by depth | 0 — berm is a barrier, not a storage feature |
| Basin | Polygon | Lower cells by depth | Area × depth × 0.8 |
| Dam | Polyline | Raise cells TO crest elevation using np.maximum | 0 — volume calculated after re-analysis from ponding layer |

Swale with companion berm: excavated volume is placed downhill with 75% compaction factor. Berm height = sqrt(cross_section × 0.75). Adds a triangular above-ground ponding wedge to swale capacity.

# 6.  Earthwork Properties Dialog

The EarthworkPropertiesDialog is a popup shown after drawing an earthwork, and again when editing one. It adapts its fields based on earthwork type.

- Parameters accepted: ew_type, geometry, earthwork (edit mode), peak_inflow_m3, crest_elevation

## 6.1  Fields by Type

| Type | Fields Shown |
| --- | --- |
| Swale | Name, Depth, Width, Companion berm checkbox, live capacity (m³ and L), berm height estimate + disclaimer, swale length, recommended length shown in red/green, peak storm inflow if placed on contour |
| Berm | Name, Depth, Width — note shown: berms are barriers with no storage capacity |
| Basin | Name, Depth, Width — live capacity updates as values change |
| Dam | Name, Crest elevation (pre-filled from DEM at endpoints), Wall thickness — note to re-analyse for volume |

Spillway section: shown in edit mode when a spillway point has been placed on the earthwork. Includes an overflow elevation spinner.

# 7.  Key Methods in terrain_flow.py

This section documents the most important methods in the main controller file. These are the methods most likely to be touched when adding new features.

## 7.1  DEM and Setup

- _set_dem(path) — loads the DEM, creates a DEMBurner instance, triggers slope layer generation

- _calculate_slope_layer() — runs gdal:slope algorithm, applies four-zone discrete colour ramp at alpha 160

## 7.2  Analysis

- _run_baseline() / _run_with_earthworks() — both call _run_analysis(dem_path, label)

- AnalysisWorker(QThread) — background thread that runs FlowAnalysis and saves the flow direction raster for catchment reuse

- _on_analysis_complete(result_paths) — first line re-enables btn_run immediately so exceptions cannot leave it permanently disabled; then loads result layers

- get_stream_threshold_cells() — converts spin_threshold_ha value to a cell count using cell_size_m2

## 7.3  Contours

- _generate_contours() — runs GDAL contour at user-specified interval; sets combo_contour_layer

- _find_optimal_swale_contours() — scans DEM and accumulation raster, picks N elevations by peak flow; exclusion zone cells are masked from candidate search via rasterio.features.rasterize; uses binary raster trick for reliable contour generation at exact elevations

- _analyse_contour_flow() — samples flow accumulation along each contour; skips contours intersecting exclusion zones; generates graduated blue layer with star markers for top 5 results

## 7.4  Exclusion Zones

- _get_exclusion_union() — returns QgsGeometry.unaryUnion() of all drawn exclusion polygons

- _apply_exclusion_to_contour_layer() — splits contours at exclusion boundaries; excluded segments go to a grey-dashed layer

- Overlap warning — when a swale is drawn over an exclusion zone, the overlapping section is highlighted in bold red

## 7.5  Earthwork Drawing

- _activate_draw_line(ew_type) — activates DrawLineTool for swale, berm, or dam

- _on_geometry_drawn(ew_type, geometry) — for dams: samples DEM at both endpoints, sets crest_elevation to max(elev_start, elev_end); then opens the properties dialog

- _sample_dem_at_point(x, y) — reads elevation from self._burner.original array at a given coordinate

## 7.6  Per-Earthwork Catchment

- _delineate_and_display_catchment() — loads the saved flow direction raster via pysheds.Grid.from_raster(), delineates catchment at the peak accumulation point, displays a teal polygon with area, slope, and inflow labels

# 8.  Processing Modules

## 8.1  scs_runoff.py — SCSRunoff

- STORM_PRESETS — dictionary of {label: (intensity_mm_hr, duration_hr)} for all storm scenarios

- cn_adjusted(cn, moisture) — applies AMC I and AMC III adjustments to the base curve number

- runoff_mm(rainfall_mm, cn) — applies the SCS equation to return effective runoff depth in mm

- catchment_volume(runoff_mm, area_m2) — converts runoff depth and catchment area to total volume in m³

## 8.2  flow_analysis.py — FlowAnalysis

- pysheds pipeline: fill_pits → fill_depressions → flowdir (D8) → accumulation

- stream_mask — derived from threshold in either contributing area (ha) or runoff volume (m³) mode

- Saves three rasters: flow direction (float32, D8 values 1/2/4/8/16/32/64/128), flow accumulation, stream network

- get_catchment_polygons() — vectorises catchment masks into polygons

## 8.3  dem_burner.py — DEMBurner

- burn_earthworks(earthworks) — applies all enabled earthworks to a copy of the original DEM array; returns the modified array

- get_ponding_layer(modified_dem) — runs pysheds depression fill on modified DEM; returns filled minus modified, clipped to values ≥ 0; this shows where water will pond

- _burn_dam() — uses np.maximum so dam cells can only be raised, never lowered; preserves terrain above crest elevation

## 8.4  earthwork.py — Earthwork + EarthworkManager

- Earthwork data class — stores type, geometry, depth, width, name, enabled state, spillway point and elevation

- EarthworkManager — maintains the list of all drawn earthworks; handles add, remove, enable/disable, serialisation

- Capacity formulas are defined here and called by the properties dialog for live preview

# 9.  Map Tools

Each map tool is a subclass of QgsMapTool and handles mouse events on the QGIS canvas.

| File | Tool | Behaviour |
| --- | --- | --- |
| draw_line_tool.py | DrawLineTool | Left-click adds vertices; right-click or double-click finishes and emits geometry |
| draw_polygon_tool.py | DrawPolygonTool | Same pattern as line tool but closes the polygon on finish |
| place_point_tool.py | PlacePointTool | Single left-click places a point and emits coordinate; used for spillway placement |
| ponding_query_tool.py | PondingQueryTool | Click inside a ponding zone polygon to calculate and display its stored volume |
| select_contour_tool.py | SelectContourTool | Click a contour line to select it; reads the elevation field from the attribute table |

# 10.  Slope Classification Layer

The slope classification layer is generated automatically when a DEM is loaded. It runs gdal:slope and applies a four-zone discrete colour ramp at 160/255 transparency so the hillshade remains visible underneath.

| Zone | Degrees | Colour | Earthwork Guidance |
| --- | --- | --- | --- |
| Gentle | 0 – 3° | Green | Ideal for spillways and gentle diversions — minimal erosion risk |
| Optimal swale | 3 – 8° | Yellow | Best zone for swale placement |
| Caution | 8 – 15° | Orange | Swale or berm — depends on soil type and rainfall severity |
| Steep | > 15° | Red | Berm or diversion drain recommended — swales not advised |

Warnings are shown in the plugin panel when an earthwork is drawn in the caution or steep zones. The layer can be toggled on and off via btn_toggle_slope_class.

# 11.  Exclusion Zones

Exclusion zones allow the user to mark areas where earthworks should not be placed — roads, buildings, property boundaries, areas outside their jurisdiction, or any other reason. The system is intentionally simple: the user draws polygons and the analysis respects them.

## 11.1  Drawing

- btn_draw_exclusion activates DrawPolygonTool

- Drawn polygons accumulate in a vector layer called Exclusion Zones

- Displayed as semi-transparent red polygons with a red dashed border

- Multiple polygons can be drawn and will all be respected

## 11.2  Effect on Analysis

- Optimal swale contour search: exclusion zone cells are masked from the candidate raster before ranking — the tool will not suggest placing swales in excluded areas

- Contour flow analysis: contours that intersect exclusion zones are skipped entirely

- Contour display: contour lines are split at exclusion zone boundaries; excluded segments appear as grey dashed lines so the user can still see the terrain

- Manual earthwork drawing: if a swale or other earthwork overlaps an exclusion zone, the overlapping section is highlighted red and a warning appears in the panel

## 11.3  Controls

- btn_clear_exclusion_zones — removes all exclusion polygons

- btn_toggle_exclusion_zones — show/hide the exclusion zone layer without deleting it

# 12.  Dependencies

| Library | Used For |
| --- | --- |
| QGIS / PyQGIS | QgsMapTool, QgsVectorLayer, QgsRasterLayer, QgsGraduatedSymbolRenderer and all map canvas operations |
| rasterio | Raster I/O and rasterio.features.rasterize for exclusion zone masking |
| pysheds | Flow direction, flow accumulation, and catchment delineation |
| numpy | Array operations throughout DEM burning and analysis |
| shapely | Geometry buffering, parallel offset, and interpolation for contour sampling |
| QGIS processing (GDAL) | gdal:slope for slope layer; gdal:contour for contour generation |

# 13.  Known Limitations

These are current known behaviours to be aware of — not bugs, but design constraints of the MVP build.

- Berm water retention is only visible after Re-analyse with Earthworks — it does not update immediately when drawn

- Dam capacity (ponded volume) is only calculated after Re-analyse — the properties dialog shows a note about this

- Companion berm backwater ponding is an estimate with a disclaimer — actual ponding depends on local slope variability

- Optimal contour elevations are filtered by exclusion zones at raster level before candidate ranking — very small exclusion zones at DEM resolution may not be detected

- btn_run is re-enabled at the top of _on_analysis_complete so any exception during processing cannot leave it permanently disabled — this is intentional defensive behaviour

# 14.  Quick Reference — Starting a Claude Code Session

When starting a new Claude Code session to continue work on TerrainFlow, paste the following at the start:

I am building a QGIS 3.40 LTR plugin called TerrainFlow. I have attached the current state document which describes all existing files, methods, widgets, and known limitations. Please read it before making any changes. Do not rewrite existing functionality — only add to or modify what is specifically requested.

Then attach or paste this document, describe what you want to add or fix, and Claude Code will have full context to work accurately without breaking existing features.